# St. Charles Parish

Department of Planning & Zoning

Land Use Report

Case Number:  PZr-2011-04

GENERAL APPLICATION INFORMATION

- Name/Address of Applicant:						Application Date: 1/21/2011

Roger Burks

P.O. Box 22

New Sarpy LA 70078

985.764.7906

- Location of Site:

914 East Harding Street, New Sarpy (currently vacant)

- Requested Action:

Rezone from R-1AM to C-2 to develop

SITE-SPECIFIC INFORMATION

- Size of Parcel:

60’ x 80’ or 4800 square feet

- Existing Land Use and Zoning:

Property is zoned R-1A(M) but currently is vacant and unused.

.

- Surrounding Land Uses and Zoning:

Property is surrounded by R-1A(M) zoning and uses on most sides, with lots immediately to the rear or southeast being vacant.

- Comprehensive Plan Specifications:

Maintain and encourage the residential character and encourage controlled commercial and industrial growth.

Promote land uses that employ adequate transitional zones between incompatible land uses, for instance, locating a light industrial zone closer to Good Hope Refinery, then a business park transition zone graduating down to a multi-family zone and to single-family closest to Ormond Estates.

Encourage the use of buffer zones as a means of protection of land uses from other incompatible uses

Require future land uses to employ adequate transition zones between incompatible land uses and encourage implementation of the Future Land Use Plan and zoning changes that help achieve that plan.

- RESIDENTIAL/COMMERCIAL/INDUSTRIAL

Maintain existing residential land uses and promote residential development.  Encourage commercial and light industrial growth.

Maintain the existing residential density and encourage new residential growth that maintains adequate buffer zones between residential and industrial uses by implementation of the Future Land Use Plan and zoning changes that help achieve that plan.

- Utilities:

Standard utilities serve the site.

- Traffic Access:

Site has 60’ frontage on E. Harding, a north-south collector that connects Airline Drive with River Road.

##### APPLICABLE REGULATIONS

- Appendix A., Zoning Ordinance, Section IV.9:

- Rezoning Guidelines and Criteria: Before the Planning & Zoning Commission recommends or the Parish Council rezones property, there should be reasonable factual proof by the proponent of a change that one or more of the following criteria are met:

- 1.	Land-use pattern or character has changed to the extent that the existing zoning no longer allows reasonable use of the proponent's property and adjacent property.  Reasonableness is defined as:

a.	Land use the same as, or similar to that existing or properties next to, or across the street from the site under consideration.

b.	Consideration of unique or unusual physical or environmental limitations due to size, shape, topography or related hazards or deficiencies.

- c.	Consideration of changes in land value, physical environment or economic aspects, which tend to limit the usefulness of vacant land or buildings.

- 2.	The proposed zoning change, and the potential of a resulting land use change, will comply with the general public interest and welfare and will not create:

- a.	Undue congestion of streets and traffic access.

b.	Overcrowding of land or overburden on public facilities such as transportation, sewerage, drainage, schools, parks and other public facilities.

- c.	Land or building usage which, is, or may become incompatible with existing character or usage of the neighborhood.

- d.	An oversupply of types of land use or zoning in proportion to population, land use and public facilities in the neighborhood.

- 3.	The proposed zoning change is in keeping with zoning law and precedent, in that:

- a.	It is not capricious or arbitrary in nature or intent.

- b.	It does not create a monopoly, or limit the value or usefulness of neighboring properties.

- c.	It does not adversely affect the reliance that neighboring property owners or occupants have placed upon existing zoning patterns.

- d.	It does not create a spot zone, that is, an incompatible or unrelated classification which would prevent the normal maintenance and enjoyment of adjacent properties.

- AND

- [III.] C-2 General commercial district-- Retail sales:

- 1.	Use Regulations:

- a.	A building or land shall be used for the following purposes:

- (1) All uses allowed in C-1 District. (Ord. No. 88-5-5, 5-16-88)

- (2) Retail sales (except auto and mobile home sales), usage, and storage

- (3) Hotels, motels and apartment hotels

- (4) [Repealed by Ord. No. 92-9-14, 9-8-92.]

- (5) Restaurants (including drive-in restaurants) and cafeterias. Specific land use requirements for restaurants serving alcoholic beverages are contained in subsection III.59. of these regulations, with further details contained within Chapter 3 of the St. Charles Parish Code of Ordinances.

- (Ord. No. 94-11-2, § V, 11-7-94)

- (6) Animal hospitals where all animals are kept inside the building

- (7) Service station

- (8) Commercial recreation facilities

- (9) Commercial greenhouses and nurseries

- (10)Commercial schools

- (11)Shops not to exceed two thousand five hundred (2,500) square feet of floor area for the repair     and servicing of the following: bicycles, radios, televisions, stereos and recorders, household, appliances, locksmith, typewriters, other similar uses

- (12) Shops not to exceed two thousand five hundred (2,500) square feet of floor area may also include the following uses: dressmakers, millinery, tailors, baking goods sales, laundry and dry, cleaners, theatres (but not the drive-in type)

- (13) Laboratories

- (14) Customary accessory uses incidental to the above uses when located on the same lot

- (15) Funeral homes (provided that a petition of no objection signed by a majority of property owners within a three hundred foot radius of the site and one hundred (100) percent of the property owners on the same street within the same block be filed with the Planning Zoning Department

- (16) Cemeteries and mausoleums, provided however that such uses shall be located on sites of at least twenty (20) acres, all graves shall be set back at least fifty (50) feet from all property lines, shall have a minimum street frontage of one hundred (100) feet and a fence or screen planting six (6) feet high shall be provided along all property lines adjoining all districts

- Cellular/communication towers. (Ord. No. 95-9-11, 9-18-95)

- (17) Other uses of similar intensity.

- (18) Mini-storage facilities (limited to one-story construction in C-2 district). (Ord. No. 98-3-17, § II, 3-23-98).

- b.	Special exception uses and structures include the following:

- (1) Dwelling units contained within the office building

- (2) Reserved

- (3) Reserved

- (4) Churches

- (5) Movie theaters

- (6) Temporary on-site construction buildings for a period of one (1) year upon approval of the Planning Director. (Ord. No. 88-9-9, 9-6-88)

- (Ord. No. 85-7-17, 7-22-85)

- c.	Special permit uses and structures include the following:

- (1) R-1A and R-1B uses upon review and approval by the Planning Commission.

- (2) R-3 uses upon review and approval by the Planning Commission and supporting resolution of 	the Council.

- (3) Office buildings for gaming operations, excluding all gaming activities, upon review and 	approval by the Planning Commission and supporting resolution of the Council.

- (4) Motor vehicle repair. Automobile sales and service on designated federal and state highways; 	body repair activities being strictly prohibited in the C-2 zoning district.

- (5) Heating and air conditioning service.

- (6) Sheet metal shops

- (7) Plumbing shops.

- (8) Green markets upon review and approval by the Planning Commission and supporting 	resolution of the Council. Such sites must possess frontage on a hard-surfaced public 	collector or arterial street.

- (9) Bingo halls, video bingo parlors, and off-track betting establishments upon review of the 	planning commission and ordinance of the St. Charles Parish Council.

- (Ord. No. 92-9-14, 9-8-92; Ord. No. 94-1-9, § III, 1-10-94; Ord. No. 94-6-6, § I, 6-6-94; Ord. No. 98-7-6, 7-6-98; Ord. No. 02-5-21, 5-20-02; Ord. No. 03-1-12, § III, 1-21-03; Ord. No. 07-10-10, § I, 10-15-07; Ord. No. 09-4-19, § 1, 4-20-09)

- 2.	Spatial Requirements:

- a.	Minimum lot size: Six thousand (6,000) square feet, minimum width - sixty (60) feet.

- b.	Minimum yard sizes:

- (1)	Front - twenty (20) feet

- (2)	Side - five (5) feet

- (3)	Rear - ten (10) feet.

- (Ord. No. 82-6-6, § 1, 6-7-82)

- (4)	Whenever property abuts a major drainage canal as defined by the Subdivision regulations the required setback for all structures shall be ten (10) feet measured from the inner boundary of such servitude or right-of-way, not withstanding any other more restrictive setbacks, this provision shall not apply to any lot of record created and existing prior to the effective date of Ordinance No. 99-12-8, December 15, 1999. (Ord. No. 08-8-9, § XI, 8-18-08)

- 3.	Transportation Requirements: Arterial

- 4.	Special Provisions:

- a.	Where any commercial use in a C-2 zoning district abuts any residential district or use, a six-foot high solid wood fence or masonry wall shall border the same and there shall be a buffer strip ten (10) feet wide designated and maintained on the site planted with plant materials acceptable for buffer zones.

- (Ord. No. 82-6-9, § III, 6-7-82; Ord. No. 98-1-3, § III, 1-5-98)

##### ANALYSIS

The applicant requests rezoning from R-1A(M) to C-2 on three lots in New Sarpy in order to develop neighborhood commercial uses.  No building permits would be issued on the lots without resubdivision as they are contiguous in single and separate ownership; however, even with resubdivision into one, the lot would not meet minimum area for the proposed zoning district which is 6,000 square feet.

- In order to receive a recommendation for approval, a rezoning request must meet all of the criteria of at least one of the tests listed in applicable regulations.  This request fails all three tests.

- The first test is to allow relief when land use character in an area has changed to the extent that current zoning no longer allows reasonable use of the property.  This area of New Sarpy is developed with a mix of site-built and mobile home residences with a number of vacant sites available for in-fill.  Permit data suggests that redevelopment is occurring in the area.  The property was developed with a residence in 1994; the permit database does not indicate when the building was moved or demolished.  With over 20 mobile home, renovation, and site-built house permits issued in the 700-900 blocks of New Sarpy, permit data suggests that a single-family use could develop on the site.  Therefore the first criteria is not met.

- The second test is to protect public infrastructure, public welfare, and the character of a neighborhood.  As the site is surrounded by R-1A(M) zoning and uses on most sides, rezoning to C-2 would be out of character with the surrounding neighborhood.  Nearby property across the street was rezoned to C2 in 2003 in order to permit expansion of a grocery store which was at the time legally non-conforming or grandfathered (that is, it existed continuously from a time before 1981 when the current zoning ordinance was adopted).  Other than that spot zone and one other legally non-conforming commercial use on E. S. Johnson, the neighborhood is developed with mobile homes, houses, parks, and a church.  For that reason, rezoning to C2 would be out of character, and the request fails the second test.

- The third test is to prevent spot zoning.  As a request for a rezoning to allow a commercial development on a site that does not meet minimum area requirements for the proposed zoning and would arguably have a difficult time meeting the requirements for fencing and landscape buffering (10’ wide on all sides), the request is for a spot zone that likely would require variances in order to meet parking, loading, landscaping, and buffering requirements. Therefore, the request fails the third test.

##### DEPARTMENTAL RECOMMENDATION

Denial