# St. Charles Parish

Department of Planning & Zoning

Land Use Report

Case Number:    PZS-99-111

GENERAL    APPLICATION INFORMATION

- Name/Address of Applicant:

Richard R. Dufrene				Herbert Dufrene

134 Pine Street				137 Pine Street

Bayou Gauche, La    70030			Bayou Gauche, La    70030

- Location of Site:

To the rear of Pine St. across Canal No. 10 (Pine Street Extension).

- Requested Action:

Construction approval of Pine Street Extension, resubdivision of Lot 901-B and a portion of 901-A of the “Sunset Drainage District” into Lots 1 through 8, Block 3, and Lots 1 through 11, Block 4 located in Section 3, T15S R20E, near Paradis, St. Charles Parish, La.

- Purpose of Requested Action:

Extension of existing Pine Street, creating 19 residential lots.

SPECIFIC SITE INFORMATION

- Size of Parcel:

8.44 acres

- Existing Land Use:

Vacant land, with two residential structures.

- Existing Zoning:

R-1A

- Surrounding Land Uses and Zoning:

R-1A zoning, primarily vacant land with sparse residential development.

- Comprehensive Plan Specifications:

“Maintain the unique rural character of the residential community along Bayou

Gauche Road and the fishing village atmosphere along Bayou Gauche.”

- Utilities:

Standard utilities are available and will serve the site.

- Floodplain Information:

X; non hazard area.

- Traffic Analysis:

The site is served by Bayou Gauche Road (LA 306).

ANALYSIS

Applicant desires construction approval to the extension of Pine Street for development of 19 single-family residential lots from an 8.44-acre parcel. A request for Preliminary Plat approval was presented to and granted by the Planning & Zoning Commission at the December 1999 meeting.

However, Pine Street does not meet subdivision regulations with respect to maximum block lengths being 1500 feet. The proposed extension of Pine Street does not provide cross streets and ends in a cul-du-sac, thereby increasing the nonconformity of the block lengths. The developer contends that there is a legitimate safety issue with regard to having a cross street in place. To the north, current development is in place. To the south, the undeveloped parcel of land is actively utilized by off-road vehicles. The developer is concerned that if a cross-street is constructed, these vehicles will travel down Pine Street to access the adjacent site. To avoid this possibility, the developer proposes dedicating an unconstructed right-of-way turning southward at the cul-du-sac that any future developer of the adjacent southern property will be responsible for constructing. This will require a waiver from the Planning & Zoning Commission and a supporting resolution by the Parish Council to the block length requirement and to the requirement that the cross-street gets paved.

The proposed subdivision plans have been found to be in conformance with all other current subdivision regulations and all required letters of no objection have been submitted to the Department of Planning & Zoning. Construction plans for this project are available for review at the office of the Planning & Zoning Department.

DEPARTMENTAL RECOMMENDATION

The Planning & Zoning Department recommends Construction approval. The Department also recommends that a waiver to the required 1500-foot block length be extended to an unconstructed but dedicated 60-foot right-of-way south of the cul-du-sac, to be constructed by future developer of adjacent lot.