# St. Charles Parish

Department of Planning & Zoning

Land Use Report

Case Number:  PZr-2006-18

GENERAL APPLICATION INFORMATION

- Name/Address of Applicant:

Troy A. Barrios

105 Tupelo Ct.

Luling, La  70070

- Location of Site:

- 12225 River Road, Luling

- Requested Action:

Rezoning from R-1A to R-3

- Purpose of Requested Action:

Townhomes

SITE-SPECIFIC INFORMATION

- Existing Land Use:

Apartment and vacant land

- Surrounding Land Uses and Zoning:

- R-1A zoning, semi-public land use across Badalamenti Drive; R-1A zoning & small lot residential land uses across Cross Street; R-3 zoning & land use across Louis the First Street.

- Comprehensive Plan Specifications:

- (Luling) “Maintain and encourage the residential character and encourage controlled commercial growth.”

- Utilities:

- Existing along Louis The First and Badalamenti; lacking sewer along Cross Street..

- Floodplain Information:

- X

- Traffic Access:

- River Road (LA 18), Louis The First Street, Cross Street, Badalamenti Drive.

##### APPLICABLE REGULATIONS

- Appendix A., Zoning Ordinance, Section IV.9:

- Rezoning Guidelines and Criteria: Before the Planning & Zoning Commission recommends or the Parish Council rezones property, there should be reasonable factual proof by the proponent of a change that one or more of the following criteria are met:

- 1.	Land-use pattern or character has changed to the extent that the existing zoning no longer allows reasonable use of the proponent's property and adjacent property.  Reasonableness is defined as:

a.	Land use the same as, or similar to that existing or properties next to, or across the street from the site under consideration.

b.	Consideration of unique or unusual physical or environmental limitations due to size, shape, topography or related hazards or deficiencies.

- c.	Consideration of changes in land value, physical environment or economic aspects, which tend to limit the usefulness of vacant land or buildings.

- 2.	The proposed zoning change, and the potential of a resulting land use change, will comply with the general public interest and welfare and will not create:

- a.	Undue congestion of streets and traffic access.

b.	Overcrowding of land or overburden on public facilities such as transportation, sewerage, drainage, schools, parks and other public facilities.

- c.	Land or building usage which, is, or may become incompatible with existing character or usage of the neighborhood.

- d.	An oversupply of types of land use or zoning in proportion to population, land use and public facilities in the neighborhood.

- 3.	The proposed zoning change is in keeping with zoning law and precedent, in that:

- a.	It is not capricious or arbitrary in nature or intent.

- b.	It does not create a monopoly, or limit the value or usefulness of neighboring properties.

- c.	It does not adversely affect the reliance that neighboring property owners or occupants have placed upon existing zoning patterns.

- d.	It does not create a spot zone, that is, an incompatible or unrelated classification which would prevent the normal maintenance and enjoyment of adjacent properties.

ANALYSIS

This request seeks to rezone a lot zoned R-1A but currently being used as a legally non-conforming R-3 use into zoning that allows R-3 land uses. The applicant intends to resubdivide the site into smaller lots for that purpose. The proposed lots meet or exceed the minimum requirements for both R-1A and the requested R-3 zoning.

In order to receive a recommendation for approval, the request must meet all of the criteria of at least one of the tests listed above. This application meets the tests of the second and third criteria.

The second test is designed to prevent overcrowding, congestion, and overburden of public infrastructure.  Along Louis The First and Badalamenti all infrastructure are in place so expanding a residential land use along those streets and in an area already residential would not compromise the capability to provide public services. With the exception of Cross Street, Parish infrastructure is in place and can accommodate the conceptual land uses that will result and the street network can accommodate the resulting land use. Any intent of developing along Cross Street would require infrastructure installation at developer expense.

The third test is designed to prevent spot zoning and negative impacts to neighbors.  Because it requests to increase the amount of R-3 residential land uses currently in place, the application does not request a spot zone. However, without rezoning approval, increasing the amount of R-3 residential uses on the site would have the effect of expanding a legally non-conforming use, which is something that the Zoning Ordinance discourages. By requesting to rezone to R-3, the applicant is attempting to accomplish the same through appropriate means. That is, to request rezoning approval.

Approval will encourage residential uses, one of the recommendations spelled out in the Comprehensive Land Use Ordinance for the community.

##### DEPARTMENTAL RECOMMENDATION

- The Department recommends  approval.